Week 2 Assignment

Introduction to Wireless Communications

Submitted by: Syed Muhammad Ibrahim

# Code explanation:

The code for this assignment is written in Python. The formula for Free Space Path Loss (FSPL) is implemented in dB using frequency in Hz and distance in metres. Separate functions have been written for FSPL, received power, link margin, and coverage distance.

The system parameters used are: transmit power of 30 dBm, antenna gains of 0 dBi for both transmitter and receiver, miscellaneous losses of 2 dB, and receiver sensitivity of -90 dBm. NumPy is used for array operations and distance range generation, while Matplotlib is used to plot the three output graphs.

Three frequencies are simulated: 2.4 GHz, 3.5 GHz, and 28 GHz, over a distance range of 10m to 10km. The results show that higher frequency causes greater path loss and therefore shorter coverage distance, which matches real-world wireless systems:

# 2.4 GHz: Coverage = 7.90 km

# 3.5 GHz: Coverage = 5.44 km

# 28 GHz: Coverage = 0.68 km